Michael Frayne

Final Project EXC – Jump and Branch

CMPEN 331 – Section 2

Abstract

This project is a culmination of 3 labs with the goal of creating a MIPS CPU. To involved breaking down the CPU process of performing one instruction into distinct section to allow for pipelining which drastically increases performance. Five pipeline stages were used to achieve this with multiple modules in each block handling different tasks. These stages included Instruction Fetch (IF), Instruction Decode (ID), Execution (EXE), Memory (MEM), Write Back (WB). This project also provides a clear understanding of MIPS CPU operations and how instructions can be handled to computed as fast as possible.

Instruction Fetch Stage (IF)

The IF stage contains four modules. It starts at the program counter (pc) that stores the current instruction that the computer is on. The pc counter is updated on the positive edge of every clock cycle based on the output of the pc adder. The pc adder increments the program count by 4 because MIPS processors are byte addressable and instructions are 4 bytes long. The program counter must therefore move four bytes to reach the start of the next instruction. The instruction memory uses the pc to determine which instruction needs to be computed. Stores instruction which are then accessed using the pc and given to the IF/ID pipeline register. This module is similar to the program counter and the three other pipeline registers in the MIPS Datapath as it only moves data to the next stage on the positive clock edge which is the behavior that allows for the synchronization that pipelining requires.

Instruction Decode Stage (ID)

After the instructions are fetched they must be decoded. MIPS has three main types of instructions that allow the 32 bits of each instruction to be interpreted differently. The first 6 bits and last 6 bits are the op code and function code respectively. The op code determines the type of instruction, (R, I, or J) and the function code determines what R-type function to perform. These two codes are given to the Control Unit which sets values for various multiplexers and modules to determine how data will flow to the following stages based on the op code and function code that determines how the instruction is supposed to work. One of the multiplexors for which the control unit determines the selector bits for determines whether rt or rd is used as the destination register. This is important for R-type and I-type instructions since I type instructions store computations between rs and an immediate value in rt while R-type instruction store computations rs and rt in rd. The regfile stores which reads and sends data from registers to the next stage and writes data to register after the writeback stage. The ID stage also contains the immediate extender module which extends an immediate 16 bit signed value to a 32 bit signed value so it can be computed with 32 bit values from registers. The final module in this stage is the ID/EXE pipeline register that moves data to the next stage of the pipeline on the positive edge of the clock.

Execute Stage (EXE)

After instructions are decoded and data is red from the registers the computations take place that execute the instruction in the EXE stage. This stage contains three different modules. The first is simply a multiplexer that determines whether the immediate value is used or register value for the second value of computation. This is determined by the Control unit based on whether the instruction is an R-type or I-type instruction. That value and another value from a register is fed into the ALU which is determined by the control unit. Once the computation is completed the result is sent to the EXE/MEM pipeline register which send the data to the next stage of the pipeline on the next positive clock edge.

Memory Stage (MEM)

The MEM stage only contains two modules. The first is the data memory module which is used for load and store instructions to send and receive data to the main memory of the computer. Weather this module loads or stores is determined by the control unit and is sent down the pipeline to be received at the same time as the data that is being stored or loaded. If data is loaded it is sent to the MEM/WB pipeline register to get sent to the final pipeline stage on the next positive clock cycle.

Writeback Stage (WB)

The final stage in the MIPS process is the write backstage. This stage only contains one multiplexor which whetherloaded data or the result of the alu is sent back to the regfile to be written to a register if needed.

Completed MIPS Pipelined CPU

The completed MIPS Datapath contains all pipeline stages combined along with hazard prevention and forwarding. These two features prevent slowdowns in the system which help speed up the completion of a set of instructions. This fille Datapath only contains two extra multiplexers that are used to forward data from the EXE, MEM, and WB stages and some updates to the control unit that check for data hazards and pause the first two stages as needed. These features complete the design of the MIPS processor outlined for the project.

Completed Pipeline with Jump and Branch (Extra Credit)

For the extra credit portion of the assignment, I implemented Branches and Jumps on top of the pipelined design with data hazard prevention. To do this I added a couple multiplexors to the design that helped change the program counter to where it was needed. The main multiplexor is a four to 1 multiplexor in the IF stage. This multiplexor chooses between the normal nextPc, or the address of a jump, equal to branch or not equal to branch. The address is determined by the Control Unit. The Control Unit was also updated to interperate jump, jump and link, jr, bne, and beq instructions.

- Verilog design code

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Create Date: 10/29/2025 07:53:28 PM

// Design Name:

// Module Name: IFID

// Project Name:

// Target Devices:

// Tool Versions:

// Description:

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

//IF Stage

module ProgramCounter(

input wire [31:0] npc,

input wire clk,

input wire wpcir,

output reg [31:0] pc

);

initial begin

pc = 32'd0;

end

always @ (posedge clk)

begin

if(wpcir==0)begin

pc <= npc;

end

end

endmodule

module PCMux(

input wire [1:0] pcsrc,

input wire [31:0] pc4,

input wire [31:0] bpc,

input wire [31:0] da,

input wire [31:0] jpccat,

output reg [31:0] npc

);

always @(*) begin

case(pcsrc)

2'b00: npc = pc4;

2'b01: npc = bpc;

2'b10: npc = da;

2'b11: npc = jpccat;

endcase

end

endmodule

module PcAdder(

input wire [31:0] pc,

output reg [31:0] pc4

);

always @(*)begin

pc4 = pc + 32'd4;

end

endmodule

/*

module InstructionMemory(

input wire [31:0] pc,

output reg [31:0] instOut

);

reg [31:0] memory [0:63];

//include more instructions that match with lab document

initial begin

memory[25] = {6'b100011,5'b00001,5'b00010,5'b00000,5'b00000,6'b000000};

memory[26] = {6'b100011,5'b00001,5'b00011,5'b00000,5'b00000,6'b000100};

memory[27] = {6'b100011,5'b00001,5'b00100,5'b00000,5'b00000,6'b001000};

memory[28] = {6'b100011,5'b00001,5'b00101,5'b00000,5'b00000,6'b001100};

memory[29] = {6'b000000,5'b00010,5'b01010,5'b00110,5'b00000,6'b100000};

end

always @(*) begin

instOut = memory[pc[7:2]];

end

endmodule

*/

module InstructionMemory ( // instruction memory, rom

input wire [31:0] pc, // rom address

output reg [31:0] instOut // rom content = rom[a]

);

reg [31:0] rom [0:63]; // rom cells: 64 words * 32 bits

// rom[word_addr] = instruction // (pc) label instruction

initial begin

rom[6'h00] = 32'h3c010000; // (00) main: lui $1, 0

rom[6'h01] = 32'h34240050; // (04) ori $4, $1, 80

rom[6'h02] = 32'h0c00001b; // (08) call: jal sum

rom[6'h03] = 32'h20050004; // (0c) dslot1: addi $5, $0, 4

rom[6'h04] = 32'hac820000; // (10) return: sw $2, 0($4)

rom[6'h05] = 32'h8c890000; // (14) lw $9, 0($4)

rom[6'h06] = 32'h01244022; // (18) sub $8, $9, $4

rom[6'h07] = 32'h20050003; // (1c) addi $5, $0, 3

rom[6'h08] = 32'h20a5ffff; // (20) loop2: addi $5, $5, -1

rom[6'h09] = 32'h34a8ffff; // (24) ori $8, $5, 0xffff

rom[6'h0a] = 32'h39085555; // (28) xori $8, $8, 0x5555

rom[6'h0b] = 32'h2009ffff; // (2c) addi $9, $0, -1

rom[6'h0c] = 32'h312affff; // (30) andi $10,$9,0xffff

rom[6'h0d] = 32'h01493025; // (34) or $6, $10, $9

rom[6'h0e] = 32'h01494026; // (38) xor $8, $10, $9

rom[6'h0f] = 32'h01463824; // (3c) and $7, $10, $6

rom[6'h10] = 32'h10a00003; // (40) beq $5, $0, shift

rom[6'h11] = 32'h00000000; // (44) dslot2: nop

rom[6'h12] = 32'h08000008; // (48) j loop2

rom[6'h13] = 32'h00000000; // (4c) dslot3: nop

rom[6'h14] = 32'h2005ffff; // (50) shift: addi $5, $0, -1

rom[6'h15] = 32'h000543c0; // (54) sll $8, $5, 15

rom[6'h16] = 32'h00084400; // (58) sll $8, $8, 16

rom[6'h17] = 32'h00084403; // (5c) sra $8, $8, 16

rom[6'h18] = 32'h000843c2; // (60) srl $8, $8, 15

rom[6'h19] = 32'h08000019; // (64) finish: j finish

rom[6'h1a] = 32'h00000000; // (68) dslot4: nop

rom[6'h1b] = 32'h00004020; // (6c) sum: add $8, $0, $0

rom[6'h1c] = 32'h8c890000; // (70) loop: lw $9, 0($4)

rom[6'h1d] = 32'h01094020; // (74) stall: add $8, $8, $9

rom[6'h1e] = 32'h20a5ffff; // (78) addi $5, $5, -1

rom[6'h1f] = 32'h14a0fffc; // (7c) bne $5, $0, loop

rom[6'h20] = 32'h20840004; // (80) dslot5: addi $4, $4, 4

rom[6'h21] = 32'h03e00008; // (84) jr $31

rom[6'h22] = 32'h00081000; // (88) dslot6: sll $2, $8, 0

// use 6-bit word address to read rom

end

always @(*) begin

instOut = rom[pc[7:2]];

end

endmodule

module IFIDPipelineRegister(

input wire [31:0] instOut,

input wire clk,

input wire wpcir,

input wire [31:0] pc4,

output reg [31:0] dpc4,

output reg [31:0] dinstOut

);

always @ (posedge clk) begin

if(wpcir==0) begin

dpc4 <= pc4;

dinstOut <= instOut;

end

end

endmodule

//ID stage

module ControlUnit(

input wire [5:0] op,

input wire [5:0] func,

input wire [4:0] rs,

input wire [4:0] rt,

input wire [4:0] mrn,

input wire mm2reg,

input wire mwreg,

input wire [4:0] ern,

input wire em2reg,

input wire ewreg,

input wire rsrtequ,

output reg [1:0]pcsrc,

output reg wpcir,

output reg wreg,

output reg m2reg,

output reg wmem,

output reg jal,

output reg [3:0] aluc,

output reg aluimm,

output reg shift,

output reg regrt,

output reg sext,

output reg [1:0] fwda,

output reg [1:0] fwdb

);

reg i_rs;

reg i_rt;

initial wpcir = 0;

initial pcsrc = 2'b00;

always @(*) begin

case(op)

6'b000000://r-types

begin

case(func)

6'b100000://add instruction

begin

wreg = 1'b1;

m2reg = 1'b0;

wmem = 1'b0;

aluc = 4'b0010;

aluimm = 1'b0;

regrt = 1'b0;

pcsrc = 2'b00;

jal =1'b0;

shift = 1'b0;

sext = 1'bx;

end

6'b100010://subtract instruction

begin

wreg = 1'b1;

m2reg = 1'b0;

wmem = 1'b0;

aluc = 4'b0110;

aluimm = 1'b0;

regrt = 1'b0;

pcsrc = 2'b00;

jal =1'b0;

shift = 1'b0;

sext = 1'bx;

end

6'b100100://AND instruction

begin

wreg = 1'b1;

m2reg = 1'b0;

wmem = 1'b0;

aluc = 4'b0000;

aluimm = 1'b0;

regrt = 1'b0;

pcsrc = 2'b00;

jal =1'b0;

shift = 1'b0;

sext = 1'bx;

end

6'b100101://OR instruction

begin

wreg = 1'b1;

m2reg = 1'b0;

wmem = 1'b0;

aluc = 4'b0001;

aluimm = 1'b0;

regrt = 1'b0;

pcsrc = 2'b00;

jal =1'b0;

shift = 1'b0;

sext = 1'bx;

end

6'b100110://XOR instruction

begin

wreg = 1'b1;

m2reg = 1'b0;

wmem = 1'b0;

aluc = 4'b0011;

aluimm = 1'b0;

regrt = 1'b0;

pcsrc = 2'b00;

jal =1'b0;

shift = 1'b0;

sext = 1'bx;

end

6'b000000://sll instruction

begin

wreg = 1'b1;

m2reg = 1'b0;

wmem = 1'b0;

aluc = 4'b0100;

aluimm = 1'b0;

regrt = 1'b0;

pcsrc = 2'b00;

jal =1'b0;

shift = 1'b1;

sext = 1'bx;

end

6'b000010://srl instruction

begin

wreg = 1'b1;

m2reg = 1'b0;

wmem = 1'b0;

aluc = 4'b0101;

aluimm = 1'b0;

regrt = 1'b0;

pcsrc = 2'b00;

jal =1'b0;

shift = 1'b1;

sext = 1'bx;

end

6'b000011://sra instruction

begin

wreg = 1'b1;

m2reg = 1'b0;

wmem = 1'b0;

aluc = 4'b1000;

aluimm = 1'b0;

regrt = 1'b0;

pcsrc = 2'b00;

jal =1'b0;

shift = 1'b1;

sext = 1'bx;

end

6'b001000://jr instruction

begin

wreg = 1'b0;

m2reg = 1'bx;

wmem = 1'b0;

aluc = 4'bxxxx;

aluimm = 1'bx;

regrt = 1'bx;

pcsrc = 2'b10;

jal =1'b0;

shift = 1'bx;

sext = 1'bx;

end

endcase

end

6'b100011://lw

begin

wreg = 1'b1;

m2reg = 1'b1;

wmem = 1'b0;

aluc = 4'b0010;

aluimm = 1'b1;

regrt = 1'b1;

pcsrc = 2'b00;

jal = 1'b0;

shift = 1'b0;

sext = 1'b1;

end

6'b101011://sw

begin

wreg = 1'b0;

m2reg = 1'b0;

wmem = 1'b1;

aluc = 4'b0010;

aluimm = 1'b1;

regrt = 1'bx;

pcsrc = 2'b00;

jal = 1'b0;

shift = 1'b0;

sext = 1'b1;

end

6'b001000://addi

begin

wreg = 1'b1;

m2reg = 1'b0;

wmem = 1'b0;

aluc = 4'b0010;

aluimm = 1'b1;

regrt = 1'b1;

pcsrc = 2'b00;

jal = 1'b0;

shift = 1'b0;

sext = 1'b1;

end

6'b001100://andi

begin

wreg = 1'b1;

m2reg = 1'b0;

wmem = 1'b0;

aluc = 4'b0000;

aluimm = 1'b1;

regrt = 1'b1;

pcsrc = 2'b00;

jal = 1'b0;

shift = 1'b0;

sext = 1'b0;

end

6'b001101://ori

begin

wreg = 1'b1;

m2reg = 1'b0;

wmem = 1'b0;

aluc = 4'b0001;

aluimm = 1'b1;

regrt = 1'b1;

pcsrc = 2'b00;

jal = 1'b0;

shift = 1'b0;

sext = 1'b0;

end

6'b001110://xori

begin

wreg = 1'b1;

m2reg = 1'b0;

wmem = 1'b0;

aluc = 4'b0011;

aluimm = 1'b1;

regrt = 1'b1;

pcsrc = 2'b00;

jal = 1'b0;

shift = 1'b0;

sext = 1'b0;

end

6'b000100://beq

begin

wreg = 1'b0;

m2reg = 1'bx;

wmem = 1'b0;

aluc = 4'b0110;

aluimm = 1'b0;

regrt = 1'bx;

if(rsrtequ==1)

pcsrc = 2'b01;

else

pcsrc = 2'b00;

jal = 1'b0;

shift = 1'b0;

sext = 1'b1;

end

6'b000101://bne

begin

wreg = 1'b0;

m2reg = 1'bx;

wmem = 1'b0;

aluc = 4'b0110;

aluimm = 1'b0;

regrt = 1'bx;

if(rsrtequ==0)

pcsrc = 2'b01;

else

pcsrc = 2'b00;

jal = 1'b0;

shift = 1'b0;

sext = 1'b1;

end

6'b001111://lui

begin

wreg = 1'b1;

m2reg = 1'b0;

wmem = 1'b0;

aluc = 4'b1001;

aluimm = 1'b1;

regrt = 1'b1;

pcsrc = 2'b00;

jal = 1'b0;

shift = 1'bx;

sext = 1'bx;

end

6'b000010://j

begin

wreg = 1'b0;

m2reg = 1'bx;

wmem = 1'b0;

aluc = 4'bxxxx;

aluimm = 1'bx;

regrt = 1'bx;

pcsrc = 2'b11;

jal = 1'b0;

shift = 1'bx;

sext = 1'bx;

end

6'b000011://jal

begin

wreg = 1'b1;

m2reg = 1'bx;

wmem = 1'b0;

aluc = 4'bxxxx;

aluimm = 1'bx;

regrt = 1'bx;

pcsrc = 2'b11;

jal = 1'b1;

shift = 1'bx;

sext = 1'bx;

end

endcase

if (wpcir) begin

wreg  = 1'b0;

m2reg = 1'b0;

wmem  = 1'b0;

jal   = 1'b0;

end

end

//stall

always @(*) begin

i_rs = (op != 6'b000010) && (op != 6'b000011);

i_rt = (op == 6'b000000) || (op == 6'b000100) || (op == 6'b000101);

if (ewreg && em2reg && (ern != 5'd0) &&((i_rs && ern == rs) || (i_rt && ern == rt))) begin

wpcir = 1'b1;

end else begin

wpcir = 1'b0;

end

end

//forward a

always @(*) begin

if (ewreg && (ern != 0) && (ern == rs))

fwda = 2'b01;

else if (mwreg && (mrn != 0) && (mrn == rs) && !mm2reg)

fwda = 2'b10;

else if (mwreg && (mrn != 0) && (mrn == rs) && mm2reg)

fwda = 2'b11;

else

fwda = 2'b00;

end

//forward b

always @(*) begin

if (ewreg && (ern != 0) && (ern == rt))

fwdb = 2'b01;

else if (mwreg && (mrn != 0) && (mrn == rt) && !mm2reg)

fwdb = 2'b10;

else if (mwreg && (mrn != 0) && (mrn == rt) && mm2reg)

fwdb = 2'b11;

else

fwdb = 2'b00;

end

endmodule

module addrLeftShift(

input wire [25:0] addr,

output reg [31:0] jpc

);

always @ (*)begin

jpc = {4'b0000,addr,2'b00};

end

endmodule

module immLeftShift(

input wire [15:0] imm,

output reg [31:0] immShift

);

always @ (*)begin

immShift = {{14{imm[15]}}, imm, 2'b00};

end

endmodule

module branchAddr(

input wire [31:0] dpc4,

input wire [31:0] immShift,

output reg [31:0] bpc

);

always @(*) begin

bpc = dpc4+immShift;

end

endmodule

module fwdaMux(

input wire [1:0] fwda,

input wire [31:0] qa,

input wire [31:0] ealu,

input wire [31:0] malu,

input wire [31:0] mdo,

output reg [31:0] da

);

always @(*) begin

case(fwda)

2'b00: da <= qa;

2'b01: da <= ealu;

2'b10: da <= malu;

2'b11: da <= mdo;

endcase

end

endmodule

module fwdbMux(

input wire [1:0] fwdb,

input wire [31:0] qb,

input wire [31:0] ealu,

input wire [31:0] malu,

input wire [31:0] mdo,

output reg [31:0] db

);

always @(*) begin

case(fwdb)

2'b00: db <= qb;

2'b01: db <= ealu;

2'b10: db <= malu;

2'b11: db <= mdo;

endcase

end

endmodule

module ImmediateExtender(

input wire [15:0] imm,

input wire sext,

output reg [31:0] dimm

);

always @(*) begin

if(sext==1)begin

dimm = {{16{imm[15]}},imm};

end

else begin

dimm = {16'b0,imm};

end

end

endmodule

module RsRteq(

input wire [31:0] da,

input wire [31:0] db,

output reg rsrtequ

);

always @(*) begin

if(da==db)begin

rsrtequ <= 1;

end

else begin

rsrtequ <= 0;

end

end

endmodule

module RegrtMultiplexer(

input wire [4:0] rt,

input wire [4:0] rd,

input wire regrt,

output reg [4:0] drn

);

always @(*) begin

if(regrt == 0)begin

drn = rd;

end

else begin

drn =rt;

end

end

endmodule

module jpcConcat(

input wire [31:0] jpc,

input wire [31:0] dpc4,

output reg [31:0] njpc

);

always @(*) begin

njpc = {dpc4[31:28], jpc[27:0]};

end

endmodule

//EXE Stage

module IDEXEPipelineRegister(

input wire wreg,

input wire m2reg,

input wire wmem,

input wire jal,

input wire [3:0] aluc,

input wire aluimm,

input wire shift,

input wire [31:0] dpc4,

input wire [31:0] da,

input wire [31:0] db,

input wire [31:0] dimm,

input wire [4:0] drn,

input wire clk,

output reg ewreg,

output reg em2reg,

output reg ewmem,

output reg ejal,

output reg [3:0] ealuc,

output reg ealuimm,

output reg eshift,

output reg [31:0] epc4,

output reg [31:0] ea,

output reg [31:0] eb,

output reg [31:0] eimm,

output reg [4:0] ern0

);

always @(posedge clk) begin

ewreg <= wreg;

em2reg <= m2reg;

ewmem <= wmem;

ejal <= jal;

ealuc <= aluc;

ealuimm <= aluimm;

eshift <= shift;

epc4 <= dpc4;

ea <= da;

eb <= db;

eimm <= dimm;

ern0 <= drn;

end

endmodule

module Epc4Addr(

input wire [31:0] epc4,

output reg [31:0] epc8

);

always @(*) begin

epc8=epc4+4;

end

endmodule

module ALUMultiplexerA(

input wire [31:0] sa,

input wire [31:0] ea,

input wire eshift,

output reg [31:0] a

);

always @(*) begin

case(eshift)

1'b1: a = sa;

1'b0: a = ea;

endcase

end

endmodule

module ALUMultiplexerB(

input wire [31:0] eimm,

input wire [31:0] eb,

input wire ealuimm,

output reg [31:0] b

);

always @(*) begin

case(ealuimm)

1'b1: b = eimm;

1'b0: b = eb;

endcase

end

endmodule

module ALU(

input wire [31:0] a,

input wire [31:0] b,

input wire [3:0] ealuc,

output reg [31:0] r

);

always @(*) begin

case(ealuc)

//AND

4'b0000: r = a & b;

//OR

4'b0001: r = a | b;

//add

4'b0010: r = a + b;

//xor

4'b0011: r = a^b;

//subtract

4'b0110: r = a - b;

//slt

4'b0111: begin

if(a<b)begin

r=1;

end

else begin

r=0;

end

end

//NOR

4'b1100: r = ~(a|b);

//sll

4'b0100: r = b<<a[4:0];

//srl

4'b0101: r = b>>a[4:0];

//sra

4'b1000: r = b>>>a[4:0];

//lui

4'b1001:r = b<<16;

endcase

end

endmodule

module ALUjalMux(

input wire ejal,

input wire [31:0] epc8,

input wire [31:0] r,

output reg [31:0] ealu

);

always @(*) begin

case(ejal)

1'b1: ealu = epc8;

1'b0: ealu = r;

endcase

end

endmodule

module forwardingUnit(

input wire [4:0] ern0,

input wire ejal,

output reg [4:0] ern

);

always @(*) begin

case(ejal)

1'b0: ern = ern0;

1'b1: ern = 5'b11111;

endcase

end

endmodule

//MEM Stage

module EXEMEMPipelineRegister(

input wire ewreg,

input wire em2reg,

input wire ewmem,

input wire [31:0] ealu,

input wire [31:0] eb,

input wire [4:0] ern,

input wire clk,

output reg mwreg,

output reg mm2reg,

output reg mwmem,

output reg [31:0] malu,

output reg [31:0] di,

output reg [4:0] mrn

);

always @(posedge clk)begin

mwreg <= ewreg;

mm2reg <= em2reg;

mwmem <= ewmem;

malu <=ealu;

di <= eb;

mrn <= ern;

end

endmodule

/*

module DataMemory(

input wire [31:0] mr,

input wire [31:0] mqb,

input wire mwmem,

input wire clk,

output reg [31:0] mdo

);

reg [31:0] memory [0:63];

initial begin

memory[0] = 32'hA00000AA;

memory[1] = 32'h10000011;

memory[2] = 32'h20000022;

memory[3] = 32'h30000033;

memory[4] = 32'h40000044;

memory[5] = 32'h50000055;

memory[6] = 32'h60000066;

memory[7] = 32'h70000077;

memory[8] = 32'h80000088;

memory[9] = 32'h90000099;

end

always @(negedge clk) begin

if(mwmem==1)begin

memory[mr[31:2]] <= mqb;

end

end

//shift mr by 2 bits if need to by byte addressable

always @(*) begin

mdo <= memory[mr[31:2]];

end

endmodule

*/

module DataMemory( // data memory, ram

input wire clk, // clock

input wire [31:0] malu, // ram address

input wire [31:0] di, // data in (to memory)

input wire we, // write enable

output reg [31:0] mdo // data out (from memory)

);

reg [31:0] ram [0:31]; // ram cells: 32 words * 32 bits

integer i;

initial begin // ram initialization

for (i = 0; i < 32; i = i + 1)

ram[i] = 0;

// ram[word_addr] = data // (byte_addr) item in data array

ram[5'h14] = 32'h000000a3; // (50) data[0] 0 + a3 = a3

ram[5'h15] = 32'h00000027; // (54) data[1] a3 + 27 = ca

ram[5'h16] = 32'h00000079; // (58) data[2] ca + 79 = 143

ram[5'h17] = 32'h00000115; // (5c) data[3] 143 + 115 = 258

// ram[5'h18] should be 0x00000258, the sum stored by sw instruction

end

always @(*) begin

mdo = ram[malu[6:2]]; // use 5-bit word address

end

always @ (posedge clk) begin

if (we) ram[malu[6:2]] = di; // write ram

end

endmodule

//WB Stage

module MEMWBPipelineRegister(

input wire mwreg,

input wire mm2reg,

input wire [4:0] mrn,

input wire [31:0] malu,

input wire [31:0] mdo,

input wire clk,

output reg wwreg,

output reg wm2reg,

output reg [4:0] wrn,

output reg [31:0] walu,

output reg [31:0] wdo

);

always @(posedge clk) begin

wwreg <= mwreg;

wm2reg <= mm2reg;

wrn <= mrn;

walu <= malu;

wdo <= mdo;

end

endmodule

module WritebackMultiplexer(

input wire [31:0] walu,

input wire [31:0] wdo,

input wire wm2reg,

output reg [31:0] wdi

);

always @(*) begin

case(wm2reg)

1'b1: wdi = wdo;

1'b0: wdi = walu;

endcase

end

endmodule

module RegisterFile(

input wire [4:0] rs,

input wire [4:0] rt,

input wire [4:0] wdestReg,

input wire [31:0] wbData,

input wire wwreg,

input wire clk,

output reg [31:0] qa,

output reg [31:0] qb

);

reg [31:0] registers [0:31];

integer i;

initial begin

for(i = 0; i<32; i = i+1) begin

registers[i] = 32'h0000;

end

end

always @(*) begin

qa = registers[rs];

qb = registers[rt];

end

always @(negedge clk) begin

if(wwreg==1)begin

registers[wdestReg] <= wbData;

end

end

endmodule

//TODO adjust data path and add wires needed

//TODO read through documentation and figure out what waveform should look like

//TODO write intro and abstract for report

//Data Path

module Datapath(

input wire clk_dp,

output wire [31:0] pc_dp,

output wire [31:0] instOut_dp,

output wire [31:0] ealu_dp,

output wire [31:0] malu_dp,

output wire [31:0] wdi_dp

);

//wire [31:0] pc_dp;

wire [31:0] pc4_dp;

wire [31:0] npc_dp;

wire wpcir_dp;

wire [31:0] dinstOut_dp;

wire [31:0] dpc4_dp;

wire [5:0] op_dp = dinstOut_dp[31:26];

wire [4:0] rs_dp = dinstOut_dp[25:21];

wire [4:0] rt_dp = dinstOut_dp[20:16];

wire [4:0] rd_dp = dinstOut_dp[15:11];

wire [5:0] func_dp = dinstOut_dp[5:0];

wire [15:0] imm_dp = dinstOut_dp[15:0];

wire wreg_dp;

wire m2reg_dp;

wire wmem_dp;

wire jal_dp;

wire aluimm_dp;

wire shift_dp;

wire sext_dp;

wire regrt_dp;

wire [3:0] aluc_dp;

wire [1:0] pcsrc_dp;

wire [31:0] qa_dp;

wire [31:0] qb_dp;

wire [31:0] da_dp;

wire [31:0] db_dp;

wire [31:0] dimm_dp;

wire [31:0] immShift_dp;

wire [31:0] bpc_dp;

wire [31:0] jpc_dp;

wire [31:0] jpccat_dp;

wire rsrtequ_dp;

wire [4:0] drn_dp;

wire [1:0] fwda_dp;

wire [1:0] fwdb_dp;

wire ewreg_dp;

wire em2reg_dp;

wire ewmem_dp;

wire ejal_dp;

wire ealuimm_dp;

wire eshift_dp;

wire [3:0] ealuc_dp;

wire [31:0] epc4_dp;

wire [31:0] ea_dp, eb_dp;

wire [31:0] eimm_dp;

wire [4:0] ern0_dp;

wire [31:0] epc8_dp;

wire [31:0] a_dp, b_dp;

wire [31:0] r_dp;

wire [4:0] ern_dp;

wire mwreg_dp;

wire mm2reg_dp;

wire mwmem_dp;

wire [31:0] di_dp;

wire [4:0] mrn_dp;

wire [31:0] mdo_dp;

wire wwreg_dp;

wire wm2reg_dp;

wire [4:0] wrn_dp;

wire [31:0] walu_dp;

wire [31:0] wdo_dp;

wire [25:0] addr_dp = dinstOut_dp[25:0];

wire [31:0] shamt_dp = {16'd0, eimm_dp[31],eimm_dp[14:0]};

// IF stage

ProgramCounter PC (

.npc(npc_dp),

.clk(clk_dp),

.wpcir(wpcir_dp),

.pc(pc_dp)

);

PCMux PCMUX(

.pcsrc(pcsrc_dp),

.pc4(pc4_dp),

.bpc(bpc_dp),

.da(da_dp),

.jpccat(jpccat_dp),

.npc(npc_dp)

);

PcAdder PC_ADD (

.pc(pc_dp),

.pc4(pc4_dp)

);

InstructionMemory IMEM (

.pc(pc_dp),

.instOut(instOut_dp)

);

// ID stage

IFIDPipelineRegister IFID (

.instOut(instOut_dp),

.clk(clk_dp),

.wpcir(wpcir_dp),

.pc4(pc4_dp),

.dpc4(dpc4_dp),

.dinstOut(dinstOut_dp)

);

ControlUnit CU (

.op(op_dp),

.func(func_dp),

.rs(rs_dp),

.rt(rt_dp),

.mrn(mrn_dp),

.mm2reg(mm2reg_dp),

.mwreg(mwreg_dp),

.ern(ern_dp),

.em2reg(em2reg_dp),

.ewreg(ewreg_dp),

.rsrtequ(rsrtequ_dp),

.pcsrc(pcsrc_dp),

.wpcir(wpcir_dp),

.wreg(wreg_dp),

.m2reg(m2reg_dp),

.wmem(wmem_dp),

.jal(jal_dp),

.aluc(aluc_dp),

.aluimm(aluimm_dp),

.shift(shift_dp),

.sext(sext_dp),

.regrt(regrt_dp),

.fwda(fwda_dp),

.fwdb(fwdb_dp)

);

addrLeftShift ALS(

.addr(addr_dp),

.jpc(jpc_dp)

);

immLeftShift ILS(

.imm(imm_dp),

.immShift(immShift_dp)

);

branchAddr BADDR(

.dpc4(dpc4_dp),

.immShift(immShift_dp),

.bpc(bpc_dp)

);

RegisterFile RF (

.rs(rs_dp),

.rt(rt_dp),

.wdestReg(wrn_dp),

.wbData(wdi_dp),

.wwreg(wwreg_dp),

.clk(clk_dp),

.qa(qa_dp),

.qb(qb_dp)

);

fwdaMux FWD_A (

.fwda(fwda_dp),

.qa(qa_dp),

.ealu(ealu_dp),

.malu(malu_dp),

.mdo(mdo_dp),

.da(da_dp)

);

fwdbMux FWD_B (

.fwdb(fwdb_dp),

.qb(qb_dp),

.ealu(ealu_dp),

.malu(malu_dp),

.mdo(mdo_dp),

.db(db_dp)

);

ImmediateExtender IMMEXT (

.sext(sext_dp),

.imm(imm_dp),

.dimm(dimm_dp)

);

RsRteq EQ (

.da(da_dp),

.db(db_dp),

.rsrtequ(rsrtequ_dp)

);

RegrtMultiplexer REGDST (

.rt(rt_dp),

.rd(rd_dp),

.regrt(regrt_dp),

.drn(drn_dp)

);

jpcConcat JPCcc(

.jpc(jpc_dp),

.dpc4(dpc4_dp),

.njpc(jpccat_dp)

);

// EXE stage

IDEXEPipelineRegister IDEXE (

.wreg(wreg_dp),

.m2reg(m2reg_dp),

.wmem(wmem_dp),

.jal(jal_dp),

.aluc(aluc_dp),

.aluimm(aluimm_dp),

.shift(shift_dp),

.dpc4(dpc4_dp),

.da(da_dp),

.db(db_dp),

.dimm(dimm_dp),

.drn(drn_dp),

.clk(clk_dp),

.ewreg(ewreg_dp),

.em2reg(em2reg_dp),

.ewmem(ewmem_dp),

.ejal(ejal_dp),

.ealuc(ealuc_dp),

.ealuimm(ealuimm_dp),

.eshift(eshift_dp),

.epc4(epc4_dp),

.ea(ea_dp),

.eb(eb_dp),

.eimm(eimm_dp),

.ern0(ern0_dp)

);

Epc4Addr EADDR(

.epc4(epc4_dp),

.epc8(epc8_dp)

);

ALUMultiplexerA ALUMUXA (

.sa(shamt_dp),

.ea(ea_dp),

.eshift(eshift_dp),

.a(a_dp)

);

ALUMultiplexerB ALUMUXB (

.eimm(eimm_dp),

.eb(eb_dp),

.ealuimm(ealuimm_dp),

.b(b_dp)

);

ALU ALU(

.a(a_dp),

.b(b_dp),

.ealuc(ealuc_dp),

.r(r_dp)

);

ALUjalMux AJM(

.ejal(ejal_dp),

.epc8(epc8_dp),

.r(r_dp),

.ealu(ealu_dp)

);

forwardingUnit F(

.ern0(ern0_dp),

.ejal(ejal_dp),

.ern(ern_dp)

);

// MEM stage

EXEMEMPipelineRegister EXEMEM (

.ewreg(ewreg_dp),

.em2reg(em2reg_dp),

.ewmem(ewmem_dp),

.eb(eb_dp),

.ealu(ealu_dp),

.ern(ern_dp),

.clk(clk_dp),

.mwreg(mwreg_dp),

.mm2reg(mm2reg_dp),

.mwmem(mwmem_dp),

.di(di_dp),

.malu(malu_dp),

.mrn(mrn_dp)

);

DataMemory DMEM (

.malu(malu_dp),

.di(di_dp),

.we(mwmem_dp),

.clk(clk_dp),

.mdo(mdo_dp)

);

// WB stage

MEMWBPipelineRegister MEMWB (

.mwreg(mwreg_dp),

.mm2reg(mm2reg_dp),

.mrn(mrn_dp),

.malu(malu_dp),

.mdo(mdo_dp),

.clk(clk_dp),

.wwreg(wwreg_dp),

.wm2reg(wm2reg_dp),

.wrn(wrn_dp),

.walu(walu_dp),

.wdo(wdo_dp)

);

WritebackMultiplexer WBMUX (

.walu(walu_dp),

.wdo(wdo_dp),

.wm2reg(wm2reg_dp),

.wdi(wdi_dp)

);

endmodule

- Test Bench Design Code

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Create Date: 10/29/2025 08:08:34 PM

// Design Name:

// Module Name: testbench

// Project Name:

// Target Devices:

// Tool Versions:

// Description:

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

//need to set up a datapath function

module testbench();

reg clk_tb;

wire [31:0] pc_tb;

wire [31:0] instOut_tb;

wire [31:0] ealu_tb;

wire [31:0] malu_tb;

wire [31:0] wdi_tb;

/*

wire [31:0] pc_tb;

wire [31:0] dinstOut_tb;

wire ewreg_tb;

wire em2reg_tb;

wire ewmem_tb;

wire ealuimm_tb;

wire [3:0] ealuc_tb;

wire [4:0] edestReg_tb;

wire [31:0] eqa_tb;

wire [31:0] eqb_tb;

wire [31:0] eimm32_tb;

wire mwreg_tb;

wire mm2reg_tb;

wire mwmem_tb;

wire [4:0] mdestReg_tb;

wire [31:0] mr_tb;

wire [31:0] mqb_tb;

wire wwreg_tb;

wire wm2reg_tb;

wire [4:0] wdestReg_tb;

wire [31:0] wr_tb;

wire [31:0] wdo_tb;

wire [31:0] wbData_tb;

*/

//Datapath datapath_tb(clk_tb,pc_tb,dinstOut_tb,ewreg_tb, em2reg_tb, ewmem_tb, ealuimm_tb, ealuc_tb, edestReg_tb, eqa_tb,eqb_tb,eimm32_tb, mwreg_tb,mm2reg_tb,mwmem_tb,mdestReg_tb,mr_tb,mqb_tb,wwreg_tb,wm2reg_tb,wdestReg_tb,wr_tb,wdo_tb,wbData_tb);

Datapath datapath_tb(

.clk_dp(clk_tb),

.pc_dp(pc_tb),

.instOut_dp(instOut_tb),

.ealu_dp(ealu_tb),

.malu_dp(malu_tb),

.wdi_dp(wdi_tb)

);

initial begin

clk_tb = 0;

end

always begin

#1

clk_tb = ~clk_tb;

end

endmodule

- Waveform

- Schematic

- I/O Planning

- Floor Planning